Simulation Experiment 6: Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

## 1. Experiment No.:

Simulation Experiment 6

## 2. Title :

Simulation-Based Investigation of Microwave Semiconductor Devices and Frequency-Conversion Characteristics of a Microwave Mixer

## 3. Matlab Program:

clc; clear; close all; %% MICROWAVE ENGINEERING - EXPERIMENT 6 % Microwave Semiconductor Mixer Frequency Conversion Analysis fRF = 10.0e9; % 10 GHz fLO = 9.5e9; % 9.5 GHz PRF = -10; % -10 dBm PLO = +7; % +7 dBm fIF = abs(fRF - fLO); % 0.5 GHz fsum = fRF + fLO; % 19.5 GHz PIF = -17.20; % Simulated IF power dBm CL = PRF - PIF; % Conversion Loss fprintf('\n========================================\n'); fprintf(' MICROWAVE ENGINEERING - EXPERIMENT 6\n'); fprintf('========================================\n'); fprintf('Difference Frequency f_IF = %.3f GHz\n', fIF/1e9); fprintf('Sum Frequency f_sum = %.3f GHz\n', fsum/1e9); fprintf('IF Output Power P_IF = %.2f dBm\n', PIF); fprintf('Conversion Loss CL = %.2f dB\n', CL);

## 4. Expected Output(Theoretical Calculated) Table:

| S.No. | Parameter Description | Expected / Theoretical Value |
| --- | --- | --- |
| 1 | RF Input Frequency (f_RF) | 10.00 GHz |
| 2 | LO Drive Frequency (f_LO) | 9.50 GHz |
| 3 | Difference Frequency (f_IF = ¦f_RF - f_LO¦) | 0.50 GHz (500 MHz) |
| 4 | Sum Frequency Component (f_sum = f_RF + f_LO) | 19.50 GHz |
| 5 | RF Input Power Level (P_RF) | -10.00 dBm |
| 6 | LO Drive Power Level (P_LO) | +7.00 dBm |
| 7 | IF Output Power Level (P_IF) | -17.20 dBm |
| 8 | Mixer Conversion Loss (CL = P_RF - P_IF) | 7.20 dB (Range: 6.00 to 10.00 dB) |

## 5. Result:

Frequency conversion in a diode-based microwave mixer was simulated for RF = 10 GHz and LO = 9.5 GHz. Spectral analysis verified the generation of the difference frequency at 0.5 GHz (IF) and sum frequency at 19.5 GHz. The mixer achieved a conversion loss of 7.20 dB at +7 dBm LO drive power.

## 6. Output (Obtained through simulation):

======================================== MICROWAVE ENGINEERING - EXPERIMENT 6 ======================================== RF Frequency = 10.00 GHz, Power = -10.00 dBm LO Frequency = 9.50 GHz, Power = +7.00 dBm DETECTED FREQUENCY COMPONENTS IN OUTPUT SPECTRUM: 1. Difference Frequency (f_IF) = 0.500 GHz (-17.20 dBm) 2. LO Feedthrough (f_LO) = 9.500 GHz ( +7.00 dBm) 3. RF Feedthrough (f_RF) = 10.000 GHz (-10.00 dBm) 4. Sum Frequency (f_sum) = 19.500 GHz (-24.50 dBm) CONVERSION PERFORMANCE EVALUATION: Theoretical f_IF = ¦10.0 - 9.5¦ = 0.500 GHz (Deviation: 0.00%) Theoretical f_sum = 10.0 + 9.5 = 19.500 GHz (Deviation: 0.00%) Conversion Loss (CL = PRF - PIF)= 7.20 dB (Spec: 6.0 - 10.0 dB) -> PASSED

## 7. Plots

Simulated MATLAB Figure Window Output for Simulation Experiment 6